Project Specification Sheet

Project Address: 140 Abbott Valley Run, Cumberland, RI
Designer: Ian E. Souza, I.E. Designs
Contact: Pat@Millerpropertiesri.com (401) 793-1519

General Building Details

- Style: Two-story single-family home

- Total Floors: Basement + 1st Floor + 2nd Floor

- Garage: Attached 2-car garage

- Exterior Siding:

  - Vinyl Clapboard

  - Vinyl Board & Batten Siding

- Roof Type: Gable & Hip roof design

- Roofing Material: Architectural Asphalt Shingles (30-year rated)

- Trim: 5½" Vinyl Corner Trim, Aluminum Coil Rake Trim (White)

- Soffit: Vented Vinyl Soffit

- Fascia: 1x8 Board

Dimensions (Approximated from plans)

- Ceiling Heights:

  - First Floor: 9 ft

  - Second Floor: 8 ft except for 9ft Trey Ceiling in master bedroom

  - Garage Slab to Ceiling: ~8 ft

- Roof Pitch (varies):

  - Front and main roof: 9:12

  - Garage and dormer: 4:12, 8:12, 10:12, 12:12 depending on location

Framing & Structural Components

- Wall Framing:

  - 2x6 Stud Walls @ 16" O.C. (Garage + Exterior)

  - 2x4 Stud Walls @ 16" O.C. (Interior)

- Joists & Rafters:

  - Floor: 2x12 Joists @ 16" O.C.

  - Ceiling: 2x10 Joists @ 16" O.C.

  - Roof: 2x12 Rafters @ 16" O.C. with Collar Ties

- Beams:

  - Multiple LVL (Laminated Veneer Lumber) Beams, including (4)-1¾"x18" and (3)-1¾"x11¼" in key areas

- Subfloor: ¾" T&G Plywood, glued and ring nailed

Foundation & Basement

- Foundation Wall: 10" Thick Concrete w/ (2)-#4 Rebar Top & Bottom

- Footings: 12"x20" Continuous Poured Concrete (2500 PSI), 48" Below Grade

- Basement Floor Slab: 4" Thick Concrete

- Foundation Damp-proofing: Applied to grade level

Insulation & Energy Efficiency

- Exterior Walls (conditioned): R-21 Batt with Vapor Barrier

- Basement Ceiling: R-30 Batt with Vapor Barrier

- Garage Walls: R-21 Insulation with Vapor Barrier

- Attic/Ceiling: R-38 Insulation

- Rim/Band Joists: Closed-cell Spray Foam

- Compliance: Must meet 2024 IECC and Rhode Island Energy Code

Construction Compliance & Notes

- All construction to conform to Massachusetts 110 MPH Exposure B wind code

- Compliance with the 2015 IRC (International Residential Code)

- Decks to be built per AWC Prescriptive Residential Deck Construction Guide

- Railings required for decks over 30" from grade

Additional Notes

- Electrical: All decks, balconies, and porches require at least one receptacle

- Finishes and Materials: To be coordinated and finalized between Owner and Contractor

- Kitchen Cabinets: Default is White Deluxe shaker style, Modifications to door style or color will incur an additional charge.

- Countertops: Quartz, Modifications to Material type will incur an additional charge.

- Light Fixtures: Recessed lighting, Fans, Exterior lighting, Landscaping lighting?

- Paint: 3 Sherwin Williams colors allowance, more than 3 will incur an additional charge.

- Hardware: Black hardware throughout (Doorknobs, Cabinet pulls)

- Flooring: 4” White oak in natural finish throughout, Tile in all bathrooms.

- Doors: Solid core 2 panel shaker

- Kitchen Appliances: High end Black stainless steel

WWW.MillerPropertiesRI.COM

Patio: 12’x20’ Paver patio in Keystone Panorama Supra Paver 3 pc. Granite Blend, Modifications to Style & Color will incur an additional charge.